NOTES ABOUT BATCH FILES TO CHECK FOR BAD HARDWARE

=================================================

I.  CHECKING FOR A BAD NETWORK CARD   (CHKNCARD.BAT)

Use the batch file CHKNCARD.BAT.  Logon to the network and put yourself

on a local drive on the machine.  Modify the netdrive set command at the

top of the file to be the netdrive you wish to test.

Then type  "Chkncard <filename>"

where <filename> is any file you have on the local drive.  A file that

copies in a few seconds (between 5 and 10) is best (not to big, not to

small).  This batch file should run forever and if it ever stops or

pauses for a significant amount of time on its own, you probably have

a bad network card on that machine. It has a counter of sorts (a series

of Xs that print on the screen) so you can do something like time how

many Xs are printed in a minute, then go away for an hour and see if

you have the appropriate number of X's on the screen when you come back

(60 times what you got in a single minute)

Counter looks like this:

Counters are:

1234567890

1000's Digit --

100's  Digit -- X

10's   Digit -- XXX

1's    Digit -- XXXXXXX

This would mean is making the 137 copy of the file.

You will have to use a control-C to stop the run, if it never hangs.

You should run this on every machine you suspect might have a bad

network card.

This file is not currently setup to work on a machine that does not

have any local drive (hard or floppy), but could be modified to do so.

II. CHECKING FOR A BAD DISC CONTROLLER  (CHKCONT.BAT)

Use the batch file CHKCONT.BAT.  Logon to the network and put yourself

on a local drive on the machine.  Modify the netdrive set command at the

top of the file to be the netdrive you wish to test.

Then type "CHKCONT <filename>"

where filename should be a very large file (>10 megabytes at least) on

the local drive.  Bad disk controller problems often only occur on files

that are very very large.  This file continues to copy a file from the

local drive to the network drive and then comparing it to a backup of

the original. If the copy and the orginal ever don't match, it is likely

you have a bad drive controller on your network.  You will need to use

a control-C to stop this run.

This only needs to be run on a single machine attached to your network.